# STM Lesson 121. Temperature, pressure and humidity sensor BME280. Part 1

Posted on June 7, 2018by Narod Stream Posted in I2C , Programming STM32 - 2 comments ↓

Programming LessonsLearn all the in-house cooking programming training ! Get the book on the e-mail!To learn moresheremetev.infoYandex.Direct

Evaluation BoardsUnique collection: selection by parameters, prices, detailed descriptions.To learn morerlocman.ruYandex.Direct

In this lesson we will, according to numerous requests, consider the sensor from Bosch Sensortec - BME280 .

This sensor measures several very important environmental values ​​- temperature, air humidity, and atmospheric pressure.

There is also a similar sensor - BMP280, which also measures these values, but only in addition to humidity. But it is much cheaper.

But since the requests were just to consider the sensor BME280 , then we will do it.

In addition to the fact that the sensor measures all these above mentioned values, it also manages to filter the noise, it has a very fine calibration of the readings, at the same time, it does all this work brilliantly and very quickly. Also, unlike previous similar sensors we have examined, this sensor performs all these measurements not only quickly, but also in wider ranges. For example, many sensors do not know how to measure humidity less than 20 percent.

The sensor looks like this (click on the image to enlarge the image)

And, before starting to program and connect this sensor, we briefly get acquainted with its main characteristics. If someone is interested to know in detail, then all the information is on the official website of Bosch Sensortec .

Until we start with how to connect this sensor to the controller. It can be connected either via the SPI bus or via the I2C bus. We will connect it via the I2C bus, since I got a module with this sensor in my hands, which is connected only through I2C and the SPI interface in it is not output from the sensor to the external contacts. This module looks like this

On the SPI bus, the sensor can be clocked at a frequency of up to 10 megahertz, and on the I2C bus it can be up to 3.4 megahertz

This module can be powered from 1.71 volts to 3.6 volts.

The sensor I / O port is powered in the range 1.2 - 3.6 V (there is a separate power supply for this purpose).

Sensor consumption current:

when measuring humidity and temperature at a sampling frequency of 1 hertz - 1.8 μA,

when measuring pressure and temperature at a sampling frequency of 1 hertz - 2.8 μA,

when measuring pressure, humidity and temperature at a sampling frequency of 1 hertz - 3.6 μA,

in sleep mode, 0.1 μA.

We see that such currents will occur only when sampling 1 hertz, without oversampling (oversampling). And if we apply any filters, then the output will have a completely different frequency, because of the filter coefficients. Therefore, in this case, these consumption currents will be observed even at a lower measurement frequency.

Measuring ranges: temperatures - -40 ... +85 ° C, humidity - 0 ... 100%, pressure - 300 ... 1100 hPa.

Of course, in different ranges, the measurement accuracy will be different.

For example, the humidity measurement accuracy of ± 3% RH is only provided in the range of 20 ... 80% RH, and the accuracy of the pressure measurement ± 1.0 hPa is in the range 300 ... 1100 hPa (that is, in the whole range). Temperature measurement accuracy ± 0.5 ° C (at 25 ° C),

The sensor sounds as follows:

Humidity: 0.07% - without oversampling, with maximum oversampling the noise is reduced to 0.02%.

Pressure: 1.3 Pa - without the use of oversampling and filters, with the maximum oversampling and filtering reduced to 0.2 Pa.

Temperature: 0.005 ° C - without the use of oversampling and filters, with the maximum oversampling and filtering reduced to 0.002 ° C.

Now let's look at the purpose of the sensor contacts

Vdd - analog power;

GND - common wire;

Vddio - powering the digital path;

SDO - digital signal output when using 4-wire SPI mode, when using I2C mode, the address bus (if connected to a common wire, then the logical zero in the address zero bit, if the power supply is logical 1);

GND - common wire;

CSB - CS contact (chip select) when using SPI mode;

SDI - digital signal input when using 4-wire SPI mode, digital input and output when using 3-wire SPI mode and I2C mode;

SCK is a digital signal clock bus.

Also, the sensor has a lot of different registers, with which we will get acquainted in detail while writing the code.

That's how the memory is organized in the sensor (click on the image to enlarge the image)

It should also be noted that the sensor can be in three modes: in normal mode ( NORMAL MODE ), in sleep mode ( SLEEP MODE ) and in forced mode ( FORCE MODE ).

Well, with the sleeping regime everything is clear. And what is the difference between normal and forced?

But this is what.

In normal mode, after reading the values ​​from the sensor, the latter remains in this mode and further, but in the compulsory mode, when we consider the values, the sensor automatically enters sleep mode. And, to us again to read the indicators, we need to re-initialize either normal or forced modes.

This chart shows us the best way

Now the connection diagram.

We will connect the sensor module to the Nucleo STM32F401RE board . Also to the information contacts I2C we connect a logic analyzer to see how our sensor will respond to commands. Then, once the reception is adjusted, we also connect the LCD2004 character display to the I2C board as well.

This is how our scheme looks without a display (click on the image to enlarge the image)

Create a new project in the Cube MX and select the controller

Turn on the controller timing

Then turn on the debugger

Turn on the I2C bus to connect the sensor

Override the legs for easy connection of the sensor

Let's at the same time immediately turn on the bus and connect the display

For this bus, we do not override anything, the feet remain the default.

We also include the USART bus to view the debugging information, but we still do not need any wires or adapters, USART works through the debug port

We also turn on the output of the LED leg

Go to System Configuration and configure the following frequencies (click on the image to enlarge the image)

Go to Configuration and configure I2C for the sensor

Just in case we turn on the interrupts, suddenly come in handy

Let's go into the settings of another I2C bus and make sure everything is right there

Let's go into the project settings, select the programming environment, the project folder and call our project

Save the project settings, generate the project for Keil, open it in it, then configure the programmer for auto-reload, set the optimization level to 1 and try to assemble the project.

If everything is going well, then we will continue.

Open the file main.c and connect the library to work with the lines

/* USER CODE BEGIN Includes */

#include <string.h>

/* USER CODE END Includes */

Let's add a global string array

/* Private variables ---------------------------------------------------------*/

char str1[100];

/* USER CODE END PV */

Create a module for our new library working with the sensor in the form of two files BME280.h and BME280.c the following contents

BME280.h :

#ifndef BME280_H_

#define BME280_H_

//------------------------------------------------

#include "stm32f4xx_hal.h"

#include <string.h>

#include <math.h>

//------------------------------------------------

//------------------------------------------------

#endif /* BME280_H_ */

BME280.c :

#include "BME280.h"

//------------------------------------------------

extern I2C_HandleTypeDef hi2c1;

extern UART_HandleTypeDef huart2;

extern char str1[100];

//------------------------------------------------

We connect the module in the file main.c

#include <string.h>

#include "BME280.h"

In the next part of the lesson, we'll start writing the sensor initialization, in which we count the identifier, the calibration data from the sensor, and also configure the timing for the Standby mode.

# STM Lesson 121. Temperature, pressure and humidity sensor BME280. Part 2

Posted on June 8, 2018by Narod Stream Posted in I2C , Programming STM32 - No Comments ↓

Programming LessonsLearn all the in-house cooking programming training ! Get the book on the e-mail!To learn moresheremetev.infoYandex.Direct

Buy robot constructorTeach your child to program. 6900 for a set of shares. OrderTo learn moredorado-uno.ruYandex.Direct

In the previous part of the lesson we got acquainted with this sensor, studied its characteristics, connection methods, its properties, and also created and configured a project for its programming.

In the BME280.h file, we add macros for the LED

#include <math.h>

//------------------------------------------------

#define LED_GPIO_PORT GPIOA

#define LED_PIN GPIO_PIN_5

#define LED_ON HAL_GPIO_WritePin(LED_GPIO_PORT, LED_PIN, GPIO_PIN_SET)

#define LED_OFF HAL_GPIO_WritePin(LED_GPIO_PORT, LED_PIN, GPIO_PIN_RESET)

#define LED_TGL HAL_GPIO_TogglePin(LED_GPIO_PORT, LED_PIN)

//------------------------------------------------

Go to the file BME280.c and add the error function

extern char str1[100];

//------------------------------------------------

void Error(void)

{

LED_OFF;

}

//------------------------------------------------

Since there is only one control LED on the board, we will extinguish it on error.

After this function, add the initialization function of the sensor, in which we first light our LED

//------------------------------------------------

void BME280_Init(void)

{

uint8_t value=0;

uint32_t value32=0;

LED_ON;

}

//------------------------------------------------

Also, after the error function, we will add write and read functions to a specific byte address for the I2C bus

//------------------------------------------------

static void I2Cx_WriteData(uint16_t Addr, uint8_t Reg, uint8_t Value)

{

HAL_StatusTypeDef status = HAL_OK;

status = HAL_I2C_Mem_Write(&hi2c1, Addr, (uint16_t)Reg, I2C_MEMADD_SIZE_8BIT, &Value, 1,0x10000);

if(status != HAL_OK) Error();

}

//------------------------------------------------

static uint8_t I2Cx_ReadData(uint16_t Addr, uint8_t Reg)

{

HAL_StatusTypeDef status = HAL_OK;

uint8_t value = 0;

status = HAL_I2C_Mem_Read(&hi2c1, Addr, Reg, I2C_MEMADD_SIZE_8BIT, &value, 1, 0x10000);

if(status != HAL_OK) Error();

return value;

}

//------------------------------------------------

We use these functions constantly, so they do not need an explanation.

Below we also add two more similar functions for writing two-byte and three-byte values

//------------------------------------------------

static void I2Cx_ReadData16(uint16_t Addr, uint8_t Reg, uint16_t *Value)

{

HAL_StatusTypeDef status = HAL_OK;

status = HAL_I2C_Mem_Read(&hi2c1, Addr, Reg, I2C_MEMADD_SIZE_8BIT, (uint8_t*)Value, 2,0x10000);

if(status != HAL_OK) Error();

}

//------------------------------------------------

static void I2Cx_ReadData24(uint16_t Addr, uint8_t Reg, uint32_t *Value)

{

HAL_StatusTypeDef status = HAL_OK;

status = HAL_I2C_Mem_Read(&hi2c1, Addr, Reg, I2C_MEMADD_SIZE_8BIT, (uint8_t*)Value, 3,0x10000);

if(status != HAL_OK) Error();

}

//------------------------------------------------

Now go to the header file BME280.h and write a macro for the address of our sensor, by which it will respond to the write and read commands, after learning it in the technical documentation

As we already, I hope, we all know, we write the address shifted to the left by 1 position

#define LED_TGL HAL_GPIO_TogglePin(LED_GPIO_PORT, LED_PIN)

//------------------------------------------------

#define BME280_ADDRESS 0xEC //BME280 I2C ADDRES (0x76<<1)

//------------------------------------------------

Back in the file BME.h and after the function I2Cx_ReadData24 we add the function of writing the byte to the register

//------------------------------------------------

void BME280_WriteReg(uint8_t Reg, uint8_t Value)

{

I2Cx_WriteData(BME280_ADDRESS, Reg, Value);

}

//------------------------------------------------

We add similar functions in order to not enter the address of the device every time you call.

Add a similar function to read a byte from the register

//------------------------------------------------

uint8_t BME280_ReadReg(uint8_t Reg)

{

uint8_t res = I2Cx_ReadData(BME280_ADDRESS,Reg);

return res;

}

//------------------------------------------------

Well, by tradition, we will read the device identifier to make sure that we have such a device, and that it normally exchanges data on the I2C bus .

Find the register address with the ID in the documentation, at the same time there and see the identifier itself

Zaydom file BME280.h and write macros to register and ID values

#define BME280_ADDRESS 0xEC //BME280 I2C ADDRES (0x76<<1)

#define BME280_REG_ID 0xD0 //BME280 ID REGISTER

#define BME280_ID 0x60 //BME280 I2C ID

Back in the file BME280.c and in the initialization function, we try to read the identifier from the sensor

LED_ON;

value = BME280_ReadReg(BME280_REG_ID);

Create a prototype of the sensor initialization function and call it in the function main () the file main.c

/* USER CODE BEGIN 2 */

BME280_Init();

/* USER CODE END 2 */

We will collect the project and we will sew the controller.

We see that the LED is still on, so no error has occurred, it's already good

Run the logical analysis program and configure the I2C bus

Also, configure channel 0 trigger on the positive front

Click on "Start" and reboot the controller. We obtain the following

Excellent! We have returned the correct ID.

Let's return to the BME280.c file in the initialization function and write the validation check, not always the logical analysis to do. At the same time we will display our ID in the terminal program

value = BME280_ReadReg(BME280_REG_ID);

sprintf(str1, "rnrnID: 0x%02Xrn", value);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

if(value !=BME280_ID)

{

Error();

return;

}

Now after assembling the code and firmware controller we have to watch this in the terminal program

Next, we must perform a software reset of the sensor, for this purpose there is such a register

To reboot the sensor, we must put a strictly defined number - 0xB6 - at this address.

Therefore, go to the header file BME280.h and add the corresponding macros there

#define BME280_ID 0x60 //BME280 I2C ID

#define BME280_REG_SOFTRESET 0xE0 //BME280 SOFT RESET REGISTER

#define BME280_SOFTRESET_VALUE 0xB6 //BME280 SOFT RESET VALUE

We return to the BME280.c file in the initialization function and reboot our sensor, and also wait for it to reset accordingly

return;

}

BME280_WriteReg(BME280_REG_SOFTRESET,BME280_SOFTRESET_VALUE);

Next, we have two ways to wait for the sensor to reboot. Either wait for a while with a margin, or something to track. Let's go on the second way. He is more professional. We consider the status register and find out if we are ready to continue working with our sensor.

Let us turn to the documentation for this

We see that in this register there are only 2 working bits. We are now interested in the second.

Let's go to the header file BME280.h and add the corresponding macros there

#define BME280_SOFTRESET_VALUE 0xB6 //BME280 SOFT RESET VALUE

#define BME280_REGISTER_STATUS 0XF3 //BME280 STATUS REGISTER

#define BME280_STATUS_MEASURING 0X08 //Running conversion

#define BME280_STATUS_IM_UPDATE 0X01 //NVM data copying

Let's return to the file BME280.c and add the function of reading the status above the initialization function

//------------------------------------------------

uint8_t BME280_ReadStatus(void)

{

//clear unuset bits

uint8_t res = BME280_ReadReg(BME280_REGISTER_STATUS)&0x09;

return res;

}

//------------------------------------------------

Together with the reading of the value from the register, we use the mask to zero the nonexistent bits

Now, in the initialization function, we wait for the desired state of the zero bit

BME280_WriteReg(BME280_REG_SOFTRESET,BME280_SOFTRESET_VALUE);

while (BME280_ReadStatus() & BME280_STATUS_IM_UPDATE) ;

Next, we need to count the calibration cells and distribute them in terms of variables, in order to later use it to correct the sensor readings. How it is done, is written in the code example in the technical documentation for the sensor. Therefore, we will not go into details of the cells.

Let's go to the header file BME280.h and add there the structure for the calibration values

#define BME280_STATUS_IM_UPDATE 0X01 //NVM data copying

//------------------------------------------------

typedef struct

{

uint16_t dig_T1;

int16_t dig_T2;

int16_t dig_T3;

uint16_t dig_P1;

int16_t dig_P2;

int16_t dig_P3;

int16_t dig_P4;

int16_t dig_P5;

int16_t dig_P6;

int16_t dig_P7;

int16_t dig_P8;

int16_t dig_P9;

uint8_t dig_H1;

int16_t dig_H2;

uint8_t dig_H3;

int16_t dig_H4;

int16_t dig_H5;

int8_t dig_H6;

} BME280_CalibData;

//------------------------------------------------

All the calibration values ​​of different types, so we need the functions to read different types of data from the corresponding registers, we will necessarily add such functions later.

Let's see these registers and how exactly the data is stored there.

The data in these registers is stored in the correct form (the lowest byte forward), with a rare exception, so we do not have to change the bytes.

Also in the header file, we'll add register data macros

#define BME280_STATUS_IM_UPDATE 0X01 //NVM data copying

//------------------------------------------------

#define BME280_REGISTER_DIG_T1 0x88

#define BME280_REGISTER_DIG_T2 0x8A

#define BME280_REGISTER_DIG_T3 0x8C

#define BME280_REGISTER_DIG_P1 0x8E

#define BME280_REGISTER_DIG_P2 0x90

#define BME280_REGISTER_DIG_P3 0x92

#define BME280_REGISTER_DIG_P4 0x94

#define BME280_REGISTER_DIG_P5 0x96

#define BME280_REGISTER_DIG_P6 0x98

#define BME280_REGISTER_DIG_P7 0x9A

#define BME280_REGISTER_DIG_P8 0x9C

#define BME280_REGISTER_DIG_P9 0x9E

#define BME280_REGISTER_DIG_H1 0xA1

#define BME280_REGISTER_DIG_H2 0xE1

#define BME280_REGISTER_DIG_H3 0xE3

#define BME280_REGISTER_DIG_H4 0xE4

#define BME280_REGISTER_DIG_H5 0xE5

#define BME280_REGISTER_DIG_H6 0xE7

//------------------------------------------------

Let's return to the file BME280.c and add a global variable of the type of our structure

extern char str1[100];

BME280_CalibData CalibData;

After the function  BME280_ReadReg we add two functions for reading double-byte values ​​from sensor registers - one for unsigned, one for signed

//------------------------------------------------

void BME280_ReadReg_U16(uint8_t Reg, uint16_t *Value)

{

I2Cx_ReadData16(BME280_ADDRESS,Reg,Value);

}

//------------------------------------------------

void BME280_ReadReg_S16(uint8_t Reg, int16_t *Value)

{

I2Cx_ReadData16(BME280_ADDRESS,Reg, (uint16_t*) Value);

}

//------------------------------------------------

After the function of reading the status  BME280_ReadStatus we add the function of reading the calibration cells and distributing them along the structure fields

BME280_ReadCoefficients

Call this function in the initialization function

while (BME280_ReadStatus() & BME280_STATUS_IM_UPDATE) ;

BME280_ReadCoefficients();

We will collect the code, we will sew the controller and in the terminal program we will look at the calibration values ​​read from the sensor

You, accordingly, will have different meanings.

Set the time of the STANDBY mode.

The sensor goes into this mode after making the measurements and there is a certain time in it, which we now will adjust by including certain bits in the CONFIG register.

Here is a description of the groups of its bits

The first group (bits 7: 5) is the settings for the duration of the inactive mode (STANDBY) in normal operation. This is the setting of our STANDBY.

Bits 4: 2 - settings for IIR filter timings.

Bit 0 - Enables the 3-wire SPI mode.

Here is a diagram of the sensor's operation in normal mode, which perfectly illustrates how the sensor goes to and returns from STANDBY. While in this state, if necessary, we read the data. Also here you can see at what time the current consumes the sensor.

Therefore, if we do not need to read data often, then it makes sense for a smaller consumption to set this timing to the maximum value.

Let's look at the assignment of the bits of the configuration register responsible for this timing

Let's go to the header file BME280.h and add there the macro for the configuration register

#define BME280_STATUS_IM_UPDATE 0X01 //NVM data copying

#define BME280_REG_CONFIG 0xF5 // Configuration register

Also add a mask and settings for the timing bits of the STANDBY mode

#define BME280_REGISTER_DIG_H6 0xE7

//------------------------------------------------

#define BME280_STBY_MSK 0xE0

#define BME280_STBY_0_5 0x00

#define BME280_STBY_62_5 0x20

#define BME280_STBY_125 0x40

#define BME280_STBY_250 0x60

#define BME280_STBY_500 0x80

#define BME280_STBY_1000 0xA0

#define BME280_STBY_10 0xC0

#define BME280_STBY_20 0xE0

Back in the file BME280.c and add the function of setting the timing of the STANDBY mode after the function BME280_ReadCoefficients

//------------------------------------------------

void BME280_SetStandby(uint8_t tsb) {

uint8_t reg;

reg = BME280_ReadReg(BME280_REG_CONFIG) & ~BME280_STBY_MSK;

reg |= tsb & BME280_STBY_MSK;

BME280_WriteReg(BME280_REG_CONFIG,reg);

}

//------------------------------------------------

Call this function in the sensor initialization function by setting the maximum time

BME280_ReadCoefficients();

BME280_SetStandby(BME280_STBY_1000);

In the next part of the lesson we will configure the filter and oversampling, and also consider the value of the air temperature from the sensor.

# STM Lesson 121. Temperature, pressure and humidity sensor BME280. Part 3

Posted on June 14, 2018by Narod Stream Posted in I2C , Programming STM32 - No Comments ↓

Programming for beginnersDownload the free book and become a real programmer. Get it now:To learn moresheremetev.infoYandex.Direct

Switches contactlessSwitches contactless capacitive type VBEYandex.Direct

In the previous part of the lesson we began to write the sensor initialization, in which the identifier was counted, the calibration data from the sensor, also set the timing for the Standby mode.

Now oversampling and filters.

I saw a few examples, and also much envied from the Arduino libraries, and decided to use these settings.

Let's start with the filter. The filter coefficient is also set using the corresponding bits in the configuration register

Let's go to the header file BME280.h and add a mask and settings for bits of the filter coefficient

#define BME280_STBY_20 0xE0

//------------------------------------------------

#define BME280_FILTER_MSK 0x1C

#define BME280_FILTER_OFF 0x00

#define BME280_FILTER_2 0x04

#define BME280_FILTER_4 0x08

#define BME280_FILTER_8 0x0C

#define BME280_FILTER_16 0x10

Let's return to the BME280.c file and add the function of filter coefficient adjustment after the function BME280_SetStandby

//------------------------------------------------

void BME280_SetFilter(uint8_t filter) {

uint8_t reg;

reg = BME280_ReadReg(BME280_REG_CONFIG) & ~BME280_FILTER_MSK;

reg |= filter & BME280_FILTER_MSK;

BME280_WriteReg(BME280_REG_CONFIG,reg);

}

//------------------------------------------------

Call this function in the initialization function of the sensor by setting the average filter factor of the readings

BME280_SetStandby(BME280_STBY_1000);

BME280_SetFilter(BME280_FILTER_4);

Next, oversamplings, which are configured separately for each type of measurement.

Over-sampling (oversampling) settings are stored in two registers.

In one register - ctrl_meas - the settings of temperature and pressure oversampling are stored

Here is the over-sampling settings for temperature readings

But the settings for oversampling of pressure readings

Also in this register there are settings for the operating mode of the sensor. We will look at them a little later.

And there is a separate register for setting up humidity overuse

Here are the settings

Let's go to the header file BME280.h and add there macros for these registers

#define BME280_SOFTRESET_VALUE 0xB6 //BME280 SOFT RESET VALUE

#define BME280_REG_CTRL_HUM 0xF2 // Humidity measure control register

#define BME280_REGISTER_STATUS 0XF3 //BME280 STATUS REGISTER

#define BME280_REG_CTRL_MEAS 0xF4 // Control register pressure and temperature measure

Also we will add macros for masks and settings of oversampling bits

#define BME280_FILTER_16 0x10

//------------------------------------------------

#define BME280_OSRS_T_MSK 0xE0

#define BME280_OSRS_T_SKIP 0x00

#define BME280_OSRS_T_x1 0x20

#define BME280_OSRS_T_x2 0x40

#define BME280_OSRS_T_x4 0x60

#define BME280_OSRS_T_x8 0x80

#define BME280_OSRS_T_x16 0xA0

#define BME280_OSRS_P_MSK 0x1C

#define BME280_OSRS_P_SKIP 0x00

#define BME280_OSRS_P_x1 0x04

#define BME280_OSRS_P_x2 0x08

#define BME280_OSRS_P_x4 0x0C

#define BME280_OSRS_P_x8 0x10

#define BME280_OSRS_P_x16 0x14

#define BME280_OSRS_H_MSK 0x07

#define BME280_OSRS_H_SKIP 0x00

#define BME280_OSRS_H_x1 0x01

#define BME280_OSRS_H_x2 0x02

#define BME280_OSRS_H_x4 0x03

#define BME280_OSRS_H_x8 0x04

#define BME280_OSRS_H_x16 0x05

We return to the file BME280.c and add the over-sampling settings functions for different readings after the function BME280_SetFilter

//------------------------------------------------

void BME280_SetOversamplingTemper(uint8_t osrs)

{

uint8_t reg;

reg = BME280_ReadReg(BME280_REG_CTRL_MEAS) & ~BME280_OSRS_T_MSK;

reg |= osrs & BME280_OSRS_T_MSK;

BME280_WriteReg(BME280_REG_CTRL_MEAS,reg);

}

//------------------------------------------------

void BME280_SetOversamplingPressure(uint8_t osrs)

{

uint8_t reg;

reg = BME280_ReadReg(BME280_REG_CTRL_MEAS) & ~BME280_OSRS_P_MSK;

reg |= osrs & BME280_OSRS_P_MSK;

BME280_WriteReg(BME280_REG_CTRL_MEAS,reg);

}

//------------------------------------------------

void BME280_SetOversamplingHum(uint8_t osrs)

{

uint8_t reg;

reg = BME280_ReadReg(BME280_REG_CTRL_HUM) & ~BME280_OSRS_H_MSK;

reg |= osrs & BME280_OSRS_H_MSK;

BME280_WriteReg(BME280_REG_CTRL_HUM,reg);

//The 'ctrl_hum' register needs to be written

//after changing 'ctrl_hum' for the changes to become effwctive.

reg = BME280_ReadReg(BME280_REG_CTRL_MEAS);

BME280_WriteReg(BME280_REG_CTRL_MEAS,reg);

}

//------------------------------------------------

I think you noticed that after entering the values ​​of the moisture oversampling in the corresponding register, we counted and re-recorded the data of register F4. We did this because there is such a requirement that the changes in the humidity oversampling register will become relevant only after recording the F4 register, which we reported in the commentary

We will configure the oversampling by calling these functions with the desired settings in the initialization function

BME280_SetFilter(BME280_FILTER_4);

BME280_SetOversamplingTemper(BME280_OSRS_T_x4);

BME280_SetOversamplingPressure(BME280_OSRS_P_x2);

BME280_SetOversamplingHum(BME280_OSRS_H_x1);

We consider the settings and see them in the terminal program

BME280_SetOversamplingHum(BME280_OSRS_H_x1);

value32 = BME280_ReadReg(BME280_REG_CTRL_MEAS);

value32 |= BME280_ReadReg(BME280_REG_CTRL_HUM) << 8;

sprintf(str1, "Measurements status: %04Xrn", value32);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

sprintf(str1, "Temperature: %srnPressure: %srnHumidity: %srn",

(value32 & BME280_OSRS_T_MSK) ? "ON" : "OFF",

(value32 & BME280_OSRS_P_MSK) ? "ON" : "OFF",

((value32 >> 8) & BME280_OSRS_H_MSK) ? "ON" : "OFF");

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

We will collect the code, we will sew the controller and see what settings we will display in the terminal program

It remains to fully initialize the sensor, we only enable the NORMAL mode.

As we recall, modes are configured in certain bits of the ctrl_meas register

Go to the header file BME280.h and add there macros for the mask and settings for the bits of the mode setting

#define BME280_OSRS_H_x16 0x05

//------------------------------------------------

#define BME280_MODE_MSK 0x03

#define BME280_MODE_SLEEP 0x00

#define BME280_MODE_FORCED 0x01

#define BME280_MODE_NORMAL 0x03

//------------------------------------------------

Let's return to the BME280.c file and add the function of setting the current mode after the function BME280_SetOversamplingHum

//------------------------------------------------

void BME280_SetMode(uint8_t mode) {

uint8_t reg;

reg = BME280_ReadReg(BME280_REG_CTRL_MEAS) & ~BME280_MODE_MSK;

reg |= mode & BME280_MODE_MSK;

BME280_WriteReg(BME280_REG_CTRL_MEAS,reg);

}

//------------------------------------------------

Enable NORMAL mode in the sensor initialization function

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

BME280_SetMode(BME280_MODE_NORMAL);

}

And by the closing parenthesis of the body of the function, we see that we have finally completed the initialization of the sensor.

The smallest thing left is to count the readings of the quantities. Well nothing. Thanks to the examples of the code in the technical documentation, we still, I hope, will cope with this.

Above the initialization function, we add for the time being the readings of the reading functions of the various values ​​measured by our sensor

//------------------------------------------------

float BME280_ReadTemperature(void)

{

float temper_float = 0.0f;

return temper_float;

}

//------------------------------------------------

float BME280_ReadPressure(void)

{

float press_float = 0.0f;

return press_float;

}

//------------------------------------------------

float BME280_ReadHumidity(void)

{

float hum_float = 0.0f;

return hum_float;

}

//------------------------------------------------

float BME280_ReadAltitude(float seaLevel)

{

float att = 0.0f;

return att;

}

//------------------------------------------------

Add for these functions the prototypes in the header file.

Also in the header file BME280.h and add macros for the registers of different sensor readings

#define BME280_REG_CONFIG 0xF5 // Configuration register

#define BME280_REGISTER_PRESSUREDATA 0xF7

#define BME280_REGISTER_TEMPDATA 0xFA

#define BME280_REGISTER_HUMIDDATA 0xFD

Let's start with the temperature.

The initial "raw" temperature value directly from the sensor after processing by the ADC block is written to the next register

It turns out that the temperature readings are recorded in 20 bits.

Also, we see that the bytes in the registers are in the reverse order - the highest byte ahead, so you'll have to make a coup.

Let's just consider this register without a coup.

To do this, return to the file BME280.c and add the function of reading the 24-bit register after the function BME280_ReadReg_S16

//------------------------------------------------

void BME280_ReadReg_U24(uint8_t Reg, uint32_t *Value)

{

I2Cx_ReadData24(BME280_ADDRESS,Reg,Value);

*(uint32_t *) Value &= 0x00FFFFFF;

}

//------------------------------------------------

Since we do not have 24-bit types, we use 32-bit ones, respectively, so we remove the extra high byte.

Let's read the register of temperature values ​​in the body of the function BME280_ReadTemperature

float temper_float = 0.0f;

uint32_t temper_raw;

BME280_ReadReg_U24(BME280_REGISTER_TEMPDATA,&temper_raw);

In the main () function of the main.c file, we add the local variables for storing the data of the various parameters of the sensor

/* USER CODE BEGIN 1 */

float tf = 0.0f, pf = 0.0f, af = 0.0f, hf = 0.0f;

/* USER CODE END 1 */

Let's go to the endless loop and call the temperature reading function, after which we add a delay of 1000 milliseconds, which is stipulated by the STANDBY mode set by us after reading the sensor readings

/* USER CODE BEGIN 3 */

tf = BME280_ReadTemperature();

sprintf(str1, "Temperature: %.3f *Crn", tf);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

HAL_Delay(1000);

}

Of course, in the terminal we are not going to see any evidence yet, since they are not yet calculated, but in the logical analysis program we will see what bytes we read from the register of temperature values. To do this, we'll compile the code and run the controller.

Here are our accepted bytes

The bytes are read, this is already good.

Now let's see how they are written into a variable. To do this, go back to the BME280.c file in the BME280_ReadTemperature function and display the raw temperature value in the terminal program there

BME280_ReadReg_U24(BME280_REGISTER_TEMPDATA,&temper_raw);

sprintf(str1, "Temperature RAW: 0x%08Xrn", temper_raw);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

Let's collect the code, we'll tell the controller and see our data

According to the data changes we see that they are turned upside down, that is, the high byte changes, the temperature can not go on an order of magnitude. Well, in principle, we expected it, because the bytes in the register are reversed, judging by the datasheet, the high byte ahead (big endian format). So we need to somehow turn the whole thing around. We have already met with this phenomenon when we programmed the chip ENC28J60.

Let's go to the header file BME280.h and add the macros of the byte permutation, at the same time also for 16-bit values, they too will be needed

#define LED_TGL HAL_GPIO_TogglePin(LED_GPIO_PORT, LED_PIN)

//------------------------------------------------

#define be16toword(a) ((((a)>>8)&0xff)|(((a)<<8)&0xff00))

#define be24toword(a) ((((a)>>16)&0x000000ff)|((a)&0x0000ff00)|(((a)<<16)&0x00ff0000))

We return to the file BME280.c and add the function of reading the 24-bit register in Big Endian format after the function BME280_ReadReg_U24

//------------------------------------------------

void BME280_ReadReg_BE_U24(uint8_t Reg, uint32_t *Value)

{

I2Cx_ReadData24(BME280_ADDRESS,Reg,Value);

*(uint32_t *) Value = be24toword(*(uint32_t *) Value) & 0x00FFFFFF;

}

//------------------------------------------------

Let's return to the function of reading and processing the temperature BME280_ReadTemperature  and change the function there

BME280_ReadReg_BE_U24(BME280_REGISTER_TEMPDATA,&temper_raw);

We will collect the code, we will sew the controller and see the result now

It's quite another matter!

Now, move it all 4 bits to the right, because that's how we store the raw temperature value and the four least significant bits do not participate in the readings

BME280_ReadReg_BE_U24(BME280_REGISTER_TEMPDATA,&temper_raw);

temper_raw >>= 4;

We will collect the code, we will sew the controller and we will look now result

All perfectly moved.

Now add a global variable for temporary storage of temperature readings

BME280_CalibData CalibData;

int32_t temper_int;

Let's return to the function BME280_ReadTemperature  and add some more local variables for intermediate calculations

uint32_t temper_raw;

int32_t val1, val2;

Then, following the example from the datasheet, we will calculate the intermediate temperature value, while without the use of a floating point

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

val1 = ((((temper_raw>>3) - ((int32_t)CalibData.dig_T1 <<1))) *

((int32_t)CalibData.dig_T2)) >> 11;

val2 = (((((temper_raw>>4) - ((int32_t)CalibData.dig_T1)) *

((temper_raw>>4) - ((int32_t)CalibData.dig_T1))) >> 12) *

((int32_t)CalibData.dig_T3)) >> 14;

temper_int = val1 + val2;

And now we will calculate the reading with the floating point, again following the recommendations of the datasheet

temper_int = val1 + val2;

temper_float = ((temper_int * 5 + 128) >> 8);

temper_float /= 100.0f;

We will collect the code, we will tell the controller and see the result of the function in the terminal program

Well, since we have read everything so well, we can output the raw readings to the terminal

sprintf(str1, "Temperature RAW: 0x%08Xrn", temper_raw);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

In the next part of the lesson we will learn how to read atmospheric pressure and air humidity from the sensor and then connect the LCD2004 display for easy monitoring of the readings.

# STM Lesson 121. Temperature, pressure and humidity sensor BME280. Part 4

Posted on June 15, 2018by Narod Stream Published in I2C , Programming STM32 - 1 comment ↓

Switches contactlessSwitches contactless capacitive type VBETo learn moreferrol.ruYandex.Direct

Story about the business programmerI tell about my business on site templates. Read more on the blogTo learn moregoodlifer.ruYandex.Direct

In the previous part of the lesson, we set up a filter and oversampling, and also read the temperature value from the sensor.

Now the pressure.

Let's see the register

Since the pressure is stored in the same way as the temperature, we pass to the corresponding BME280_ReadPressure function and we will read the raw pressure indications there in a similar way, but again, without fail following the requirements of the technical documentation, we first read the temperature, because the temperature readings are involved in calculating the atmospheric pressure

float press_float = 0.0f;

uint32_t press_raw, pres_int;

BME280_ReadTemperature(); // must be done first to get t_fine

BME280_ReadReg_BE_U24(BME280_REGISTER_PRESSUREDATA,&press_raw);

press_raw >>= 4;

I think you can in the terminal program, the raw indicators do not look and do not spend for this time, everything will be fine, I've checked it many times.

Therefore, first we will add local variables for intermediate calculations

uint32_t press_raw, pres_int;

int64_t val1, val2, p;

And then we calculate the pressure, following the example from the datasheet

press_raw >>= 4;

val1 = ((int64_t) temper_int) - 128000;

val2 = val1 * val1 * (int64_t)CalibData.dig_P6;

val2 = val2 + ((val1 * (int64_t)CalibData.dig_P5) << 17);

val2 = val2 + ((int64_t)CalibData.dig_P4 << 35);

val1 = ((val1 * val1 * (int64_t)CalibData.dig_P3) >> 8) + ((val1 * (int64_t)CalibData.dig_P2) << 12);

val1 = (((((int64_t)1) << 47) + val1)) * ((int64_t)CalibData.dig_P1) >> 33;

if (val1 == 0) {

return 0; // avoid exception caused by division by zero

}

p = 1048576 - press_raw;

p = (((p << 31) - val2) * 3125) / val1;

val1 = (((int64_t)CalibData.dig_P9) * (p >> 13) * (p >> 13)) >> 25;

val2 = (((int64_t)CalibData.dig_P8) * p) >> 19;

p = ((p + val1 + val2) >> 8) + ((int64_t)CalibData.dig_P7 << 4);

pres_int = ((p >> 8) * 1000) + (((p & 0xff) * 390625) / 100000);

press_float = pres_int / 100.0f;

Let's go to the main () function of the main.c file and in the endless loop also read and display the pressure in different values

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

pf = BME280_ReadPressure();

sprintf(str1, "Pressure: %.3f Pa; %.3f hPa; %.3f mmHg\r\n", pf, pf/1000.0f, pf *0.000750061683f);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

HAL_Delay(1000);

We will collect the code, we will sew the controller and see the result of the code work of the reading function and the calculation of atmospheric pressure

Now let's return to the BME280.c file and write the body of the function of calculating the altitude above sea level (this value depends on the atmospheric pressure, although, I confess, this dependence, as I noted, is unstable, or rather, the output is the altitude)

float att = 0.0f;

float atm = BME280_ReadPressure();

att = 44330.0 * (1.0 - pow(atm / seaLevel, 0.1903));

Now we need some more coefficients.

Let's go to the header file BME280.h and add macros

#define LED_TGL HAL_GPIO_TogglePin(LED_GPIO_PORT, LED_PIN)

//------------------------------------------------

#define SEALEVELPRESSURE_HPA (1013.25)

#define SEALEVELPRESSURE_PA (1013250)

//------------------------------------------------

Let's go to the main () function of the main.c file and in the infinite loop also compute and display the height above sea level

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

af = BME280_ReadAltitude(SEALEVELPRESSURE_PA);

sprintf(str1, "Altitude: %.3f m\r\n", af);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

HAL_Delay(1000);

We will collect the code, we will sew the controller and see the result in the terminal program

It remains for us to read only the humidity.

To do this, go back to the BME280.c file in the BME280_ReadHumidity function and try to read the raw value from the register that we have for the two-byte

float hum_float = 0.0f;

int16_t hum_raw;

BME280_ReadTemperature(); // must be done first to get t_fine

BME280_ReadReg_S16(BME280_REGISTER_HUMIDDATA,&hum_raw);

sprintf(str1, "Humidity RAW: 0x%08X\r\n", hum_raw);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

Also here we see that we need to read the temperature necessarily, since its value also affects the calculation of humidity.

Let's go to the main () function of the main.c file and in an infinite loop call the function of reading and calculating the air humidity

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

hf = BME280_ReadHumidity();

sprintf(str1, "Humidity: %.3f %%\r\n", hf);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

HAL_Delay(1000);

We will collect the code, we will sew the controller and see the result of reading the two-byte value in the terminal program

As we can see, we are not the same byte again.

So go into the BME280.c file and add the corresponding read function there after the function BME280_ReadReg_S16

//------------------------------------------------

void BME280_ReadReg_BE_S16(uint8_t Reg, int16_t *Value)

{

I2Cx_ReadData16(BME280_ADDRESS,Reg,(uint16_t*)Value);

*(uint16_t *) Value = be16toword(*(uint16_t *) Value);

}

//------------------------------------------------

Now we return to the function BME280_ReadHumidity  and change the called function

BME280_ReadReg_BE_S16(BME280_REGISTER_HUMIDDATA,&hum_raw);

Let's add some local variables for intermediate calculations

int16_t hum_raw;

int32_t hum_raw_sign, v_x1_u32r;

Let's assign the read value to a 32-bit sign variable and look in the terminal program for it already

BME280_ReadReg_BE_S16(BME280_REGISTER_HUMIDDATA,&hum_raw);

hum_raw_sign = ((int32_t)hum_raw)&0x0000FFFF;

sprintf(str1, "Humidity RAW: 0x%08X\r\n",hum_raw_sign);

Let's collect the code, we'll tell the controller and see the value

Now at us the younger byte dances - so should be.

Also, following the recommendations of the technical documentation, calculate the value of humidity in the value of a floating point

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

v_x1_u32r = (temper_int - ((int32_t)76800));

v_x1_u32r = (((((hum_raw_sign << 14) - (((int32_t)CalibData.dig_H4) << 20) -

(((int32_t)CalibData.dig_H5) * v_x1_u32r)) + ((int32_t)16384)) >> 15) *

(((((((v_x1_u32r * ((int32_t)CalibData.dig_H6)) >> 10) *

(((v_x1_u32r * ((int32_t)CalibData.dig_H3)) >> 11) + ((int32_t)32768))) >> 10) +

((int32_t)2097152)) * ((int32_t)CalibData.dig_H2) + 8192) >> 14));

v_x1_u32r = (v_x1_u32r - (((((v_x1_u32r >> 15) * (v_x1_u32r >> 15)) >> 7) *

((int32_t)CalibData.dig_H1)) >> 4));

v_x1_u32r = (v_x1_u32r < 0) ? 0 : v_x1_u32r;

v_x1_u32r = (v_x1_u32r > 419430400) ? 419430400 : v_x1_u32r;

hum_float = (v_x1_u32r>>12);

hum_float /= 1024.0f;

We will collect the code, we will tell the controller and we will see the result

All indications are perfectly read and updated!

You can now delete the display of the raw value

sprintf(str1, "Humidity RAW: 0x%08X\r\n", hum_raw_sign);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

It remains for us to connect the display and display our readings on it, since it is more beautiful with the display than in the terminal program, and more autonomously, since it will not be necessary to connect the circuit to a PC.

For this we copy the files lcd.h and lcd.c from the lesson  project 105 NRF24_RX_LCD to our project.

In the file main.c we connect this library

#include "BME280.h"

#include "lcd.h"

Also connect the file  lcd.c to the project tree.

Let's move to the file and remove the declaration of the string variable, since it is not needed there

char str1[100];

Back in the main.c file and in the main () function, we call the display initialization

/* USER CODE BEGIN 2 */

LCD_ini();

BME280_Init();

In an endless cycle, we display the various indicators on our display by inserting in the appropriate places the appropriate code

tf = BME280_ReadTemperature();

sprintf(str1, "Temperature: %.3f *C\r\n", tf);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

LCD_SetPos(0, 0);

sprintf(str1, "%11.3f *C", tf);

LCD_String(str1);

pf = BME280_ReadPressure();

sprintf(str1, "Pressure: %.3f Pa; %.3f hPa; %.3f mmHg\r\n", pf, pf/1000.0f, pf *0.000750061683f);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

LCD_SetPos(0, 1);

sprintf(str1, "%11.3f hPa", pf/1000.0f);

LCD_String(str1);

LCD_SetPos(0, 2);

sprintf(str1, "%11.3f mmHg", pf * 0.000750061683f);

LCD_String(str1);

af = BME280_ReadAltitude(SEALEVELPRESSURE_PA);

sprintf(str1, "Altitude: %.3f m\r\n", af);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

hf = BME280_ReadHumidity();

sprintf(str1, "Humidity: %.3f %%\r\n", hf);

HAL_UART_Transmit(&huart2,(uint8_t*)str1,strlen(str1),0x1000);

LCD_SetPos(0, 3);

sprintf(str1, "%7.3f %% %4.1f m", hf, af);

LCD_String(str1);

HAL_Delay(1000);

It remains to collect the code, flash the controller, disable the logic analyzer and connect the display.

We admire the result of the work of our code (click on the image to enlarge the image)

Thus, today we have mastered yet another excellent weather sensor, which with high accuracy and speed can measure both temperature and pressure, and humidity of air.

Thank you all for your attention!